Smithers Bridge

Smithers, WV

Summary of Site Visit – 08.22.08

General Information- See attachment #1

- Concrete T-beam construction

- 3- spans = 48’ each

- Simply supported on concrete piers

- Skew = 19°

- Bridge spans a creek and a roadway

- ADT = 7600

- Roadway width = 42’

Observations of the Condition – See attachment #2

- Bridge is posted

- Middle span appears to have rotated/shifted

- Damage is prevalent mainly at piers

  - Connection to beams

  - Underside of pier

- Roadway has cracking at piers

- Rest of superstructure appears in fairly good condition

Access and Other Considerations for Testing

- Underside access is very good for two spans

- Access to the midspan is slightly more difficult because of the creek

- Roadway under the bridge will have to be closed (scaffolding)

- Bridge will likely have to remain open to emergency vehicles

Preliminary Modeling and Load Rating – See attachment #3

- Preliminary SAP2000 model

  - Frame and Shell Elements

  - Simple Supports

  - No Modeling of the Piers

- Preliminary ABAQUS Model

  - Solid Elements

  - Fixed Supports

  - Piers Included

Attachment #1 – Structural Plan and Site Elevation

Attachment #2 – Photo Summary of Condition

Underside of the Superstructure – Very Little Damage

Extensive Damage Underside of Pier

Damage beneath Beam Seat

Cracking in Roadway above Piers

Attachment #3 – Modeling Summary

CAD Model of the Bridge and Site

Simple SAP Model of Superstructure

Simple ABAQUS Model Including Piers